# FactoryPulse Machine Failure Reference

Synthetic plant scenario | Admin and trainer reference | Version 1

Use this reference with the admin prompt to create a plant-supervisor exercise in Scenario Lab. All machines, thresholds, lots, orders and outcomes are invented. The trainer-only incident key is at the end; keep it out of learner-visible fields before diagnosis.

# Learner mission and workflow

A simulated press slows while dimensional rejects rise and a dispatch cutoff approaches. The learner compares independent condition evidence, contains suspect output and chooses a maintenance and production response.

| Node | System | Role | Feeds |
| --- | --- | --- | --- |
| sensors | S-204 vibration, current and temperature | Engineering | machine |
| machine | M-204 press condition | Engineering | production |
| schedule | Shift plan and M-205 spare capacity | Operations | production |
| production | Units complete and queue | Operations | quality |
| quality | Dimensional inspection and lot hold | Safety | shipments |
| shipments | Dispatch staging and cutoff | Operations | End |

Dependency path: sensors → machine → production → quality → shipments; schedule → production. The incident begins at the sensor/condition boundary and propagates downstream.

# Sensor and condition observations

The chart PDF visualizes this trend. Portable-meter readings are intermittent; a blank means no independent reading was taken.

| Time | S-204 mm/s | Meter mm/s | Current A | Temp C |
| --- | --- | --- | --- | --- |
| 08:00 | 2.1 | 3.1 | 42 | 61 |
| 09:00 | 2.2 | — | 44 | 64 |
| 10:00 | 2.1 | 5.2 | 46 | 68 |
| 11:00 | 2.3 | — | 50 | 73 |
| 12:00 | 2.4 | 7.6 | 55 | 81 |
| 13:00 | 2.3 | 8.0 | 58 | 85 |

Invented review lines: vibration 6.0 mm/s, motor current 54 A and motor temperature 78 C. A normal S-204 display is not enough to clear the machine.

# Maintenance and inspection evidence

| Time | Record | Observation |
| --- | --- | --- |
| 07:40 | WO-204-CAL | Scheduled S-204 calibration marked complete; dashboard status green. |
| 10:00 | WO-204-X1 | Portable vibration 5.2 mm/s versus S-204 display 2.1 mm/s. |
| 12:00 | WO-204-X2 | Portable vibration 7.6 mm/s; motor temperature 81 C. |
| 12:20 | WO-204-X3 | Bearing noise and roughness noted in a safe spot check. |

| Lot | Time | Examined | Rejects | Rate | Disposition |
| --- | --- | --- | --- | --- | --- |
| 401-A | 09:00 | 100 | 2 | 2% | Released |
| 401-B | 10:00 | 100 | 3 | 3% | Trend review |
| 401-C | 11:00 | 100 | 5 | 5% | Hold advised |
| 401-D | 12:00 | 100 | 9 | 9% | Hold and inspect |
| 401-E | 13:00 | 100 | 11 | 11% | Hold and inspect |

Invented reject review line: over 4% in an hourly lot. Incoming material RM-77 passed a 30-of-30 sample. Poor material remains a plausible hypothesis to check, but the available evidence does not support it.

# Production and shipment commitments

| Resource | Plan | Constraint |
| --- | --- | --- |
| M-204 | 120 units/hour | Main press; output falls after 11:00. |
| M-205 | 70 units/hour | 30 units/hour already committed; 40 spare. |

| Order | Lot | Units due | Target | Machine |
| --- | --- | --- | --- | --- |
| ORD-401 | LOT-401 | 360 | 13:15 | M-204 |
| ORD-402 | LOT-402 | 240 | 15:00 | M-204 |

| Shipment | Order | Units | Cutoff | Status at 12:00 |
| --- | --- | --- | --- | --- |
| SHIP-401 | ORD-401 | 240 | 13:45 | Staging short; recent lot needs review. |
| SHIP-402 | ORD-402 | 180 | 16:00 | Not yet staged. |

At 12:00, 180 units remain in the M-204 queue. There is no new demand. Carrier slot CARR-09 is confirmed available. Reallocation to M-205 protects some urgent units but cannot replace all M-204 throughput.

# Decision design and trainer only incident key

The learner should inspect independent readings, investigate M-204, contain suspect lots, repair the source and consider limited M-205 reallocation or shipment replanning. Demand spike, raw material and carrier delay are decoys. Score evidence-backed diagnosis, timely containment, quality protection and illustrative late units against a same-workload no-action run.

Trainer-only cause: WO-204-CAL saved incorrect vibration display gain 0.06 and zero 1.9 instead of the intended training coefficients 1.0 and 0.0. The mapping flattened S-204's trend while M-204 bearing wear increased, reducing throughput and raising rejects. Keep this out of the title, summary, mission and success fields. Reveal it only after diagnosis.

Before publication, confirm a connected six-node workflow, solvable evidence, plausible decoys, a hidden cause, safe generic inspect/repair/reroute actions and no personal details in generated learner content. The simulator does not operate a real machine; costs and outcomes are illustrative.